## wash lighting

### General

#### The fixture shall be an Altman Q-LITE as manufactured by Altman Stage Lighting, or approved equal. Fixtures that do not meet the following performance criteria in this specification will not be acceptable.

#### The fixture shall be a tungsten halogen based wash light, designed as a compact, broad beam flood light for stage, studio, and location lighting.

#### The fixture shall utilize a 300 to 1000 Watt maximum tungsten halogen linear lamp.

#### Unit shall be IP20 rated for indoor use.

#### The fixture shall be cETLus listed, and shall be so labeled when delivered to site. The fixture shall be cETLus listed under the UL1598 standard.

#### Ambient operating temperature of 32°F to 104°F (-0°C to 40°C).

#### Normal operation of optical and control functions of the fixture shall not require tools.

### Physical

#### The fixture shall have a sheet steel housing and major components, free of defects such as burrs, pits and malformations.

#### Finish shall be Epoxy Sandtex black, electrostatic application.

#### Fixture shall not weigh more than 7 lbs. (3.18 Kgs.).

#### Physical dimensions of the luminaire shall be 6.75-inches (171.5mm) [H] x 11.5-inches (292.1mm) [W] x 6.75-inches (171.5mm) [D] (when barn doors installed, barn doors are optional and sold separately)

#### Fixture shall be supplied with two (2) accessory mounting screws on the face of the unit.

#### Fixture shall have dual-locking-dog tilt handles – one on each side of the housing.

#### The recessed single contact socket shall be easily accessible for re-lamping of double-ended tungsten halogen lamps.

#### Fixture shall feature a front end accessory slot with a latching lock. Gel slot shall accept the following accessories:

##### Barn doors

##### Snoot

##### Color frame

##### Safety screen

#### Fixture shall be supplied with:

##### Steel yoke constructed from rigid flat steel

##### 18-inch safety cable

##### Color frame/snoot

##### Safety screen

### Electrical

#### The fixture shall be equipped with 120VAC or 230VAC leads and sockets for operation, lamp dependent.

### Optical

#### The optical assembly shall consist of a peened reflector, formed to provide broad uniform horizontal and vertical illumination.

#### The light output of the luminaire shall have a horizontal beam angle of 102º and a vertical beam angle of 130º. Horizontal field angle shall be 132º. Vertical field angle shall be 140º and shall produce 56 foot-candles center beam at 10 feet with FCM lamp at rated lumens.

###### END SPECIFICATION

©Altman Lighting 2018